| Purpose: | This document provides guidance on fire prevention. |
| --- | --- |
| Policy/ Principles: | Workplace supervisors are expected to identify areas and processes likely to present a fire risk and to take preventative measures. |
| Scope: | The areas in which there is a great than normal risk of fire |
| Procedures: Staff member Staff Member Supervisors | Precautions Against the Outbreak of Fire There are three essential factors, sometimes called 'the fire triangle', which must be present before a fire can break out - remove one of these three factors and the fire will go out. The control of these three essentials is the basis of all fire prevention and control. Control the potential fuel supply Use non flammable materials, materials of low combustibility, fire proofed materials etc., Limit the amount of flammable materials in any specific location that has not been designed and designated for their storage. Limit the supply of oxygen (air) Keep automatic fire doors clear Keep manual fire doors closed, not propped open Note: Positive limitation of oxygen as a means of fire prevention is normally available only in industrial or laboratory situations. The exclusion of the oxygen support medium is, however, an important fire extinguishing method. Fire doors limit the flow of air to a fire. Heat limitation Avoid any form of intense radiant heat if possible. Do not overload electrical circuits as this leads to overheating and fire risk. Reduction of Fire Risk Keep furniture, clothing and other combustible materials at a safe distance from radiant heat. Careless placing of portable heaters is a major source of fire. Used non-flammable, low toxicity solvents for cleaning office machines, rather than methylated spirits or lighter fuel. Do not use passageways and fire exits for the storage of materials. This also applies to office and laboratory equipment. Keep solvent (flammable liquids) supplies in laboratories to a minimum, preferably not more than will be needed during the current working shift. Keep solvents which are not in immediate use, in a specially designated solvent storage cabinet. Place used solvents in the waste solvent container(s) for recovery or disposal. These solvents should be collected and put in a safe place daily. DO NOT put solvents down the drain. This practice is both a serious explosion hazard and a contravention of NSW Environmental statutes. DO NOT place solvents in refrigerators in open containers. Explosive concentrations of vapour/air mixtures can form from very small amounts of solvent, which may be ignited by sparks from micro switches and thermostats. Laboratory refrigerators should have all spark sources placed outside the cold chamber. Classification of Fires Decide on the selection of fire extinguishing equipment based on the type of fire risk. Fires are classified as follows: Class A; Ordinary combustibles e.g. wood, paper. Class B; Flammable and combustible liquid e.g. petrol, mentholated spirits. Class C; Flammable gases e.g. propane. Class D; Combustible metals e.g. Aluminium Powder. Class E; Fires which include an electrical hazard. Class F; Cooking Oils and fats. Note: Class D fires require special purpose extinguishers., Types of Fire Extinguishing Equipment Canvas hoses. These are used by Fire Brigade. Their use by untrained persons could result in serious or even fatal injuries to the users or other persons in the immediate vicinity. |
| Procedures: | Hose reels are connected to the mains supply. They are operated by turning on the control tap next to the reel. The hose may then be unreeled as required and the jet controlled by the operator similarly to a domestic hose. One person can operate this type of hose. Blankets extinguish by the mechanical smothering action. Woven fibre blankets are light in weight, flexible and can be re-used after cleaning. Blankets are best used on small fires and in combination with an extinguisher. Portable extinguishers (see Table 1) |
| Procedures Supervisor OFM | Selection of Portable Fire Extinguishers Select a suitable extinguishers for a given situation depending on the following factors (refer also to Table 1): - nature of the fire. - construction and occupancy of the building; - property to be protected; - availability of other fire fighting equipment; - expected delay before a fire brigade can arrive; and - expertise of the users. Note: Fire extinguishers for use in vehicles must be fitted with a hose to assist in gaining access to awkward locations. Installation of Portable Fire Extinguishers Locate the extinguisher in a conspicuous and readily accessible position and its location and type shall be clearly indicated by standard signs. Install the fire extinguisher that is supported by a suitable and substantial hook or bracket, fastened to a wall, partition or other structure as shown below Note: An extinguisher may be installed in an unlocked cabinet which is at a suitable height and is clearly marked. When the extinguisher may be subject to unauthorised interference, the cabinet may be locked, as long as it has a glass panel front which in the event of fire can be broken to permit removal of the extinguisher. |
| OFM | Inspection of Portable Fire Extinguishers Keep a record of fire fighting equipment in your area e.g Laboratory, Workshop Inspect every extinguisher every 6 months. Advise Division/Office on commencement of inspection and maintenance and the details of the contractor. Ask the contractor to report to a responsible member of the Division/ Office prior to the inspection and maintenance operation. Provide the Division/Office a copy of the report on completion of the inspection and maintenance operation. |
| Definitions: | OFM - Office of Facility Management |
| References: | Australian Standard AS 1470 - Health and Safety at Work, Principles and Practices Australian Standard AS 1319 - Safety Signs for the Occupational Environment Australian Standard AS 2444 - Portable Fire Extinguishers, Selection and Location Australian Standard AS 1851.1 - Maintenance of Fire Protection Equipment, Portable Fire Extinguishers Australian Standard AS 1851.1 - Maintenance of Fire Protection Equipment, Fire Hose Reels |

| Source/Contact: Manager, Manager Staff Health |  | Process Owner/Publisher: Occupational Health and Safety |  |
| --- | --- | --- | --- |
| Created: Amended: | July 2006 May 2008 | Status: Issued | Version 2.0 |